POS_SDK Development Manual

(V 1.3)

Content

Abstract	7

1.1 Nouns Explanation	7

Chapter 2. Function illustration	8

2.1 POS Constant definition list	8

2.2 TSPL Constant definition list	10

2.3 CPCL Constant definition list	10

2.4 POS Function list	11

2.5 TSPL Function list	12

2.6 CPCL Function list	14

2.7 Function invoking example	14

2.8 POS Function illustration	18

LONG  POS_Port_OpenA/W(LPSTR szName, INT iPort, BOOL bFile, LPSTR szFilePath)	18

LONG  POS_Port_Close(LONG iPrinterID)	18

LONG  POS_Output_PrintFontStringA/W(LONG iPrinterID, INT iFont, INT iThick, INT iWidth, INT iHeight, INT iUnderLine, LPCSTR lpString)	19

LONG  POS_Control_SetRotaryPrint(LONG iPrinterID,INT n)	20

LONG  POS_Control_SetInvertedPrint(LONG iPrinterID,INT n)	21

LONG  POS_Control_OppositeColor(LONG iPrinterID, BOOL bOppsite)	21

LONG  POS_Control_AlignType(LONG iPrinterID, LONG iAlignType)	22

LONG  POS_Output_PrintOneDimensionalBarcodeA/W(LONG iPrinterID, INT iType, INT iWidth, INT iHeight, INT hri, LPCSTR lpString)	23

LONG  POS_Output_PrintTwoDimensionalBarcodeA/W(LONG iPrinterID, INT iType, INT parameter1, INT parameter2, INT parameter3, LPCSTR lpString)	24

LONG  POS_Output_SendLocalFileA/W(LONG iPrinterID, LPCSTR lpFilePath)	25

LONG  POS_Output_DownloadRamBmpA/W(LONG iPrinterID, LPCSTR lpFilePath)	25

LONG  POS_Output_PrintRamBmp(LONG iPrinterID,INT n)	26

LONG  POS_Output_PrintBmpDirectA/W(LONG iPrinterID, LPCSTR lpFilePath)	27

LONG  POS_Output_PrintBmpDirectA_POS76(LONG iPrinterID, LPCSTR lpFilePath)	27

LONG  POS_Output_PrintStringA/W(LONG iPrinterID, LPSTR lpString)	28

LONG  POS_Output_PrintFlashBmp(LONG iPrinterID,INT n)	28

LONG  POS_Control_PrintTestpage(LONG iPrinterID)	28

LONG  POS_Output_PrintData(LONG iPrinterID, LPCSTR lpBuffer, INT iLength)	29

LONG  POS_Control_SetPrintPosition(LONG iPrinterID, LONG iLeftMargin, LONG iWidth)	30

LONG  POS_Control_SetLineSpace(LONG iPrinterID, INT iSpace)	31

LONG POS_Control_SetPrintFontE(LONG iPrinterID,BOOL iFont,BOOL iBold,BOOL iDoubleWidth,BOOL iDoubleHeight, BOOL  iUnderLine)	32

LONG POS_Control_SetPrintFontC(LONG iPrinterID,BOOL iDoubleWidth,BOOL iDoubleHeight,BOOL iUnderLine)	33

LONG  POS_Control_CutPaper(LONG iPrinterID, LONG iType, LONG iLines)	33

LONG  POS_Control_FeedLines(LONG iPrinterID, LONG iLines)	34

LONG  POS_Control_CashDraw(LONG iPrinterID, LONG iPort, LONG iTime1, LONG iTime2)	34

LONG  POS_Control_BlackMark(LONG iPrinterID)	35

LONG  POS_Status_RTQueryStatus(LONG iPrinterID)	35

LONG  POS_Status_RTQueryTypeStatus(LONG iPrinterID,INT n)	36

LONG  POS_Status_QueryTaskStatus(LONG iPrinterID,INT second)	38

LONG  POS_Input_PrinterId(LONG iPrinterID, char *buf)	39

LONG  POS_Control_ReSet(LONG iPrinterID)	39

LONG  Color24_GrayBW(LPCTSTR szSourceFile,LPCTSTR szTargetFile)	40

VOID  ASBCallback(int status, void * param)	40

LONG  POS_Status_ASB(LONG iPrinterID, INT n, ASBCallBack p, void * pDlgOwer)	40

LONG POS_Control_SetFontSize (LONG iPrinterID,  INT iWidthSize,INT iHeightSize)	43

LONG  POS_Control_SetTabs (LONG iPrinterID ,LPCSTR pszPosition,INT count)	44

LONG POS_Control_ExecuteTabs (LONG iPrinterID)	45

2.9  TSPL Function illustration	46

LONG  SetIs21();	46

LONG  PageSetupTSPL(LONG iPrinterID, INT PageWidth, INT PageHeight);	46

LONG  DrawLineTSPL(LONG iPrinterID,INT StartX, INT StartY, INT LineWidth, INT LineHeight);	47

LONG  PrintTSPL21(LONG iPrinterID,INT Set);	49

LONG  PrintTSPL51(LONG iPrinterID,INT Set,INT Copy);	49

LONG  DrawBorderTSPL(LONG iPrinterID,INT LineWidth, INT top_left_x, INT top_left_y, INT bottom_right_x, INT bottom_right_y);	50

LONG DrawTextTSPL(LONG iPrinterID,INT start_x, INT start_y, BOOL isSimplifiedChinese, INT xMultiplication,INT yMultiplication, INT rotate, CString content);	51

LONG DrawBarCodeTSPL(LONG iPrinterID,INT start_x, INT start_y, CString type, INT height, BOOL isReadable,INT rotate, INT narrowWidth, INT wideWidth, CString content);	53

LONG  ClearBuffTSPL(LONG iPrinterID);	55

LONG  GetPrinterStatusTSPL(LONG iPrinterID)	55

LONG  DriveBeepTSPL(LONG iPrinterID)	56

LONG  SetPaperbackOrPaperFeedTSPL(LONG iPrinterID,BOOL isFeedBack, INT mDot)	57

LONG  ReverseAreaTSPL(LONG iPrinterID,INT start_x, INT start_y, INT width, INT height)	58

LONG  SetGAPTSPL(LONG iPrinterID,DOUBLE value)	59

LONG  SetLabelReferenceTSPL(LONG iPrinterID,INT x,INT y)	60

LONG  DownLoadBitMapTSPL(LONG iPrinterID,BOOL isMoveFlash,CString PathName)	61

LONG  Draw2DBarCodeTSPL(LONG iPrinterID,INT itype, INT start_x, INT start_y, INT para1, INT para2, INT para3, LPCSTR content)	63

LONG  PutBitMapTSPL(LONG iPrinterID,INT start_x, INT start_y,CString fileName)	65

LONG  DrawBitMapTSPL(LONG iPrinterID,INT start_x, INT start_y, INT mode, LPCSTR fileName)	67

LONG  SetCharsetNameTSPL(LONG iPrinterID,CString CharsetName)	68

LONG  SelectCodePageTSPL(LONG iPrinterID,INT value)	69

2.10  CPCL Function illustration	71

LONG CPCL_Print(LONG iPrinterID)	71

LONG CPCL_Form(LONG iPrinterID)	72

LONG CPCL_PageSetup(LONG iPrinterID,INT offset, INT height, INT qty, INT width)	72

LONG CPCL_DrawLine(LONG iPrinterID, INT x0, INT y0, INT x1, INT y1, INT width)	73

LONG CPCL_DrawBox(LONG iPrinterID, INT x0, INT y0, INT x1, INT y1, INT width)	74

LONG  CPCL_DrawText(LONG iPrinterID, INT value, INT font, INT vsize, INT hsize, INT x, INT y, LPCSTR data)	75

LONG CPCL_InverseLine(LONG iPrinterID, INT x0, INT y0, INT x1, INT y1, INT width)	77

LONG CPCL_SetAlign(LONG iPrinterID, INT align)	78

LONG CPCL_SetBold(LONG iPrinterID, INT value)	78

LONG CPCL_SetInverseText(LONG iPrinterID, INT value)	79

LONG CPCL_SetSpacing(LONG iPrinterID, INT spacing)	80

LONG CPCL_SetUnderLineText(LONG iPrinterID, INT value)	80

LONG CPCL_DrawBarcode(LONG iPrinterID, INT value, LPCSTR type, INT width, INT ratio, INT height, INT x, INT y, LPCSTR data)	81

LONG CPCL_SetHRI(LONG iPrinterID, LPCSTR offset)	82

LONG  CPCL_Draw2Barcode_QR(LONG iPrinterID, INT x, INT y, INT Mn, INT Un,  LPCSTR Sn,  LPCSTR data)	83

LONG CPCL_Draw2Barcode_PDF417(LONG iPrinterID, INT x, INT y, INT XDn, INT YDn,  INT Cn, INT Sn, LPCSTR content)	84

LONG CPCL_PrintBMP(LONG iPrinterID, INT type,INT x, INT y, LPCSTR lpFilePath)	85

LONG CPCL_SetPageRotate(LONG iPrinterID, INT value)	85

Appendix	86

1. CODE128	summarize	86

2. Character set	87

Version illustration

| Version | SDK version | Amendment record | Write | Time |
| --- | --- | --- | --- | --- |
| V1.0 | V1007 and higher | This document is the assort document of V1007 version and higher, which don’t compatible with previous versions | Wang Xin | 2016-6-30 |
| V1.1 | V1008 and higher | This document is based on development manual V1.0, adding TSPL command description. | Zhang Tongli | 2017-1-20 |
| V1.2 | V1010 and higher | 1 Modify the serial port of POS_Port_OenA/W interface to support flow control 2 Add SetTimeOut interface to set timeout time (except download bitmap interface) 3 Add SetDownLoadBmp to set the download bitmap timeout interface 4AddPOS_Output_PrintBmpDirectA_POS76 pin printer type direct print bitmap interface | Su Bo | 2020-4-27 |
| V1.3 | V1011 and higher | 1 In the POS function part, add POS_Status_ASB to set the automatic status return; add POS_Control_SetFontSize to set the font magnification; add POS_Control_SetTabs to set the horizontal tab position; add POS_Control_ExcuteTabs to execute tabs. 2 Added CPCL commands description | Zhang Xiang Rang | 2022-3-18 |

# Abstract

POS_SDK is printer quadratic element development package provided for our POS printer and label printer model, which bases on Windows platform, including printer interface dynamic link library POS_SDK.ddl、Demo routine source code、POS_SDK development manual. POS_SDK is interface calling function and printing solution provided for development of third-party software.

Printer interface dynamic link library POS_SDK.ddl mainly package the on/off operate performance function of serial interface, parallel interface, USB interface and Ethernet interface. It also packages the ESC/POS instruction set performance function and performance function of sending and receiving data, uploading local file and printing .

Demo routine source code provide routine source code of POS_SDK.dll for users to refer in secondary development.

POS_SDK development manual is used in POS_SDK application illustration. Please read user’s manual carefully for your convenience use.

Chapter 1. Printer Relevant Knowledge

- 1.1 Nouns Explanation

- Printing Width:The maximum horizontal print range that printer supports, decided by printer itself. For example, as for 80mm printer, the printing width is 72mm(576 dots),  while as for 58mm printer, the printing width is 48(384 dots).

- Printing Area: Printing area can be set by commands, and it must be less than or equal to printing width.

- Line Height: The height of characters line. Line Height = Characters Height + Lines Space.

- Black Mark Paper: Black mark is black patch pre-printed on the paper, users can use it to locate printing position.

- Dpi(dots per inch): Printing dots on each inch paper

- Vertical or Horizontal Moving Units: A moving unit is a printing dot by default, the horizontal distance is 1/8mm and the vertical distance is 1/8mm.

- 1.2 Relevant Knowledge

- Western Language Printing: Western characters we called including ASCII characters and Codepages by default. The ASCII characters range is 0x20~0x7F, the Codepage range is 0x80~0xFF. Western language (such as German and Spanish) have their own single byte codepage. Because Codepage code and Chinese code have overlapping part, please print Codepage content in Western language printing mode. Common Western language characters fonts are: Font A: 12x24 Font B: 9x17.

- Chinese Printing: The Chinese characters including simplified Chinese and traditional Chinese by default. The common simplified Chinese character set we use is GB2312 and GB18030, while the common traditional Chinese character set we use is BIG5. Common Chinese character font: 24x24 (dots).

- Twice Height Printing: In this printing mode, the height of character is twice of normal height of character.

- Twice Width Printing: In this printing mode, the width of character is twice of normal width of character.

- TSPL command: Feature of the models support TSPL command is that buffer similar with canvas will be built in the printer, the printed content could specify coordinate on the canvas, only call printer method will make the printer print content, otherwise does not print.

# Chapter 2. Function illustration

- POS Constant definition list

| Classification | Statement | Value | Illustration |
| --- | --- | --- | --- |
| Function returned status | #define POS_ES_PAPERENDING | 6L | Paper will run out |
|  | #define POS_ES_DRAWERHIGH | 5L | High level signal of cash draw |
|  | #define POS_ES_CUT | 4L | Cutter non-recover |
|  | #define POS_ES_DOOROPEN | 3L | Paper case open |
|  | #define POS_ES_HEAT | 2L | Print head overheat |
|  | #define POS_ES_PAPEROUT | 1L | Paper out |
|  | #define POS_ES_SUCCESS | 0L | successfully/send successfully/status normally/printing finished |
|  | #define POS_ES_INVALIDPARA | -1L | Parameter error |
|  | #define POS_ES_WRITEFAIL | -2L | Failed to write |
|  | #define POS_ES_READFAIL | -3L | Failed to read |
|  | #define POS_ES_NONMONOCHROMEBITMAP | -4L | not monochrome bitmap |
|  | #define POS_ES_OVERTIME | -5L | overtime/write overtime/read overtime/printing unfinished |
|  | #define POS_ES_FILEOPENERROR | -6L | Failed to open pic/file |
|  | #define POS_ES_OTHERERRORS | -100L | Error caused by other reason |
| 1D barcode | #define POS_BT_UPCA | 4001L | UPC-A |
|  | #define POS_BT_UPCE | 4002L | UPC-E |
|  | #define POS_BT_JAN13 | 4003L | JAN13 (EAN13) |
|  | #define POS_BT_JAN8 | 4004L | JAN 8 (EAN8) |
|  | #define POS_BT_CODE39 | 4005L | CODE39 |
|  | #define POS_BT_ITF | 4006L | ITF |
|  | #define POS_BT_CODABAR | 4007L | CODABAR |
|  | #define POS_BT_CODE93 | 4073L | CODE93 |
|  | #define POS_BT_CODE128 | 4074L | CODE128 |
| 2D barcode | #define POS_BT_PDF417 | 4100L | PDF417 |
|  | #define POS_BT_DATAMATRIX | 4101L | Data Matrix |
|  | #define POS_BT_QRCODE | 4102L | QR Code |
| Barcode character position | #define POS_HT_NONE | 4011L | Not print |
|  | #define POS_HT_UP | 4012L | Print above the barcode |
|  | #define POS_HT_DOWN | 4013L | Print under the barcode |
|  | #define POS_HT_BOTH | 4014L | Print both above and under the barcode |

- 2.2 TSPL Constant definition list

| Function returned status | #define TSPL_SUCCESS | 0L | Normal |
| --- | --- | --- | --- |
|  | #define TSPL_IDERROR | -1L | Printer handle error |
|  | #define TSPL_PARAM_LESS_EQUAL_ZERO | -2L | Parameter under or equal to zero |
|  | #define TSPL_PARAM_GREAT_RANGE | -3L | Parameter above specified range |
|  | #define TSPL_PARAM_INVALID | -4L | Parameter Invalid |
|  | #define TSPL_PRINTER_STATUS_OUTPAPER | 1L | Printer out of paper |
|  | #define TSPL_PRINTER_STATUS_WORK | 2L | Printing |
|  | #define TSPL_PRINTER_STATUS_ENCLOSURENOCLOSE | 3L | Printer case open |
|  | #define TSPL_PRINTER_STATUS_ERROR | 4L | Printer internal error |
| 2D barcode type | #define TSPL_BT_PDF417 | 4100L | PDF417 |
|  | #define TSPL_BT_DATAMATRIX | 4101L | DATAMATRIX |
|  | #define TSPL_BT_QRCODE | 4102L | QRCODE |

- 2.3 CPCL Constant definition list

| Function returned status | #define CPCL_SUCCESS | 0L | Normal |
| --- | --- | --- | --- |
|  | #define CPCL_IDERROR | -1L | Printer handle error |
|  | #define CPCL_WRITEFAIL | -2L | WRITEFAIL |
|  | #define CPCL_READFAIL | -3L | READFAIL |
|  | #define CPCL_PARAM_INVALID | -4L | Parameter Invalid |
|  | #define CPCL_OVERTIME | -5L | Overtime |
|  | #define CPCL_OTHERERRORS | -100L | Other errors |
| 2D barcode type | #define CPCL_BT_PDF417 | 4100L | PDF417 |
|  | #define CPCL_BT_DATAMATRIX | 4101L | DATAMATRIX |
|  | #define CPCL_BT_QRCODE | 4102L | QRCODE |

- 2.4 POS Function list

| Function classification | Functional function | Illustration |
| --- | --- | --- |
| Port operation | 1. POS_Port_OpenA/W | Open port |
|  | 2. POS_Port_Close | Close port |
| Print | 1. POS_Output_PrintFontStringA/W | print formatting character |
|  | 2. POS_Control_SetRotaryPrint | Rotating printing |
|  | 3. POS_Control_SetInvertedPrint | Invert printing |
|  | 4. POS_Control_OppositeColor | Reverse printing |
|  | 5. POS_Control_AlignType | Align type |
|  | 6. POS_Output_PrintOneDimensionalBarcodeA/W | 1D barcode printing |
|  | 7. POS_Output_PrintTwoDimensionalBarcodeA/W | 2D barcode printing |
|  | 8. POS_Output_SendLocalFileA/W | Print local file |
|  | 9. POS_Output_DownloadRamBmpA/W | Bitmap download |
|  | 10. POS_Output_PrintRamBmp | Print downloaded bitmap |
|  | 11. POS_Output_PrintBmpDirectA/W | Drive mode printing |
|  | - 12. POS_Output_PrintBmpDirectA_POS76 | Dot Matrix type drive printing |
|  | 13. POS_Output_PrintStringA/W | Print character string |
|  | 14. POS_Output_PrintFlashBmp | Print pre-downloaded bitmap |
|  | 15. POS_Control_PrintTestpage | Print test page |
|  | 16. POS_Output_PrintData | Print buffer data |
|  | 17. POS_Control_SetPrintPosition | Set print position |
|  | 18. POS_Control_SetLineSpace | Set line space |
|  | 19. POS_Control_SetPrintFontE | Western font print format |
|  | 20. POS_Control_SetPrintFontC | Chinese font print format |
|  | - 21.POS_Control_SetFontSize | Set character size |
| Machine control | 1. POS_Control_CutPaper | Control printer cut paper |
|  | 2. POS_Control_FeedLines | Paper feeding |
|  | 3. POS_Control_CashDraw | Control printer cash draw |
|  | 4. POS_ Control_BlackMark | Black mark position |
| Status query | 1. POS_Status_RTQueryStatus | paper status checking |
|  | 2. POS_Status_RTQueryTypeStatus | printer status checking |
|  | 3. POS_Status_QueryTaskStatus | Printing finished demonstration |
|  | 4. POS_Input_PrinterId | Get printer ID |
|  | 5.ASBCallback | User-provided callback function |
|  | 6.POS_Status_ASB | Set automatic status return |
| others | 1. POS_Control_ReSet | Initialize printer |
|  | 2. Color24_GrayBW | 24 bitmap convert to monochrome bitmap |
|  | 3.SetTimeOut | Set the timeout time (except bit download bitmap interface) |
|  | 4.SetDownLoadBmpTimeOut | Set the download bitmap timeout |
| illustration | The functions which have “A/W” in the rear of function name support Unicode character encoded mode. If using ANSI encode mode standard, please use function with “A”; if using Unicode encode mode, please use function with “W” |  |

- 2.5 TSPL Function list

| Function classification | Functional function | Illustration |
| --- | --- | --- |
| Print | 1. LONG PageSetupTSPL | Set page width and page height |
|  | 2. LONG DrawLineTSPL | Draw line |
|  | 3. LONG PrintTSPL21 | TL21 print buffer |
|  | 4. LONG PrintTSPL51 | TL51 print buffer |
|  | 5. LONG DrawBorderTSPL | Draw rectangle |
|  | 6. LONG DrawTextTSPL | Print words |
|  | 7. LONG DrawBarCodeTSPL | 1D barcode print |
|  | 8. LONG Draw2DBarCodeTSPL | 2D barcode print(TL21 not supported now) |
|  | 9. LONG ClearBuffTSPL | Clear printer buffer |
|  | 10. LONG DownLoadBitMapTSPL | Bitmap download(TL21 not supported now) |
|  | 11. LONG PutBitMapTSPL | Put downloaded bitmap into printer buffer (TL21 not supported now) |
|  | 12. LONG ReverseAreaTSPL | Reverse printing on specified area |
|  | 13. LONG SetGAPTSPL | Set the vertical spacing between labels |
|  | 14. LONG SetLabelReferenceTSPL | Defines the origin position of the label |
|  | 15. LONG SetCharsetNameTSPL | Set the international character set (TL21not supported now) |
|  | 16. LONG SelectCodePageTSPL | Choose characters codepage(TL51 not supported now) |
|  | 17.DrawBitMapTSPL | Draw bitmap |
| Machine control | 1. LONG DriveBeepTSPL | Drive buzzer beep for one sound |
|  | 2. LONG SetPaperbackOrPaperFeedTSPL | Feed and back paper |
| Status query | 1. LONG GetPrinterStatusTSPL | Status Check |
| illustration | 1. LONG SetIs21 | Used to confirm it is TL21 model or not, for convenience to distinguish command and related condition |

- 2.6 CPCL Function list

| Function classification | Functional function | Illustration |
| --- | --- | --- |
| Print | 1.LONG CPCL_Print | Print buffer |
|  | 2.LONG CPCL_Form | The printer feeds the paper to the beginning of the next label page |
|  | 3.LONG CPCL_PageSetup | Set parameters such as label size |
|  | 4.LONG CPCL_DrawLine | Line drawing commands |
|  | 5.LONG CPCL_DrawBox | Frame commands |
|  | 6.LONG CPCL_DrawText | Print text |
|  | 7.LONG CPCL_InverseLine | Inverted area (in the way of drawing a line, the area that the line passes through is inversely displayed) |
|  | 8.LONG CPCL_SetAlign | Set the alignment of the current line content |
|  | 9.LONG CPCL_SetBold | Set font bold |
|  | 10.LONG CPCL_SetInverseText | Set the characters to be highlighted |
|  | 11.LONG CPCL_SetSpacing | Set character spacing |
|  | 12.LONG CPCL_SetUnderLineText | Set whether to print underline |
|  | 13.LONG CPCL_DrawBarcode | Printing 1D barcodes |
|  | 14.LONG CPCL_SetHRI | HRI character command (set 1D barcode text to print automatically) |
|  | 15.LONG CPCL_Draw2Barcode_QR | Print QR code |
|  | 16.LONG CPCL_Draw2Barcode_PDF417 | Print PDF417 QR code |
|  | 17.LONG CPCL_PrintBMP | Printing Monochrome Bitmaps |
|  | 18.LONG CPCL_SetPageRotate | Rotate the entire page 90 degrees clockwise |
| illustration | When you want to call the CPCL_SetPageRotate function, you must set the width parameter in the CPCL_PageSetup function, otherwise the rotation will not work. |  |

- 2.7 Function invoking example

- POS Function invoking example

//take USB interface printer as example

//invoking procedure:open port→send printing content→close port:

…

LONG  m_hPrinter;//efine port return handle

LONG  ret;//efine port print function return value variable

…

//invoke POS_Port_OpenA to open print port

m_hPrinter = POS_Port_OpenA(“SP-USB1”, 1002, false,null);

//identify opening port successfully or not

if(m_hPrinter < 0)

{

POS_Port_Close(m_hPrinter);

MessageBox(_T("open failed"));

return;

}

else

{

MessageBox(_T("open successfully"));

return;

}

…

//send printing content

ret = POS_Output_PrintFontStringA(m_hPrinter,0,1,1,1,0, “Hi, thank you for choosing our printer, We will get your the best experience!\r\n”);

//identify sending printing content successfully or not

switch (ret)

{

case POS_ES_SUCCESS:

MessageBox(_T("sending successfully"));

break;

case POS_ES_INVALIDPARA:

MessageBox(_T("Parameter error"));

break;

case POS_ES_WRITEFAIL:

MessageBox(_T("sending failed"));

break;

case POS_ES_OVERTIME:

MessageBox(_T("overtime"));

break;

case POS_ES_OTHERERRORS:

MessageBox(_T("other error"));

break;

}

…

POS_Port_Close(m_hPrinter); //close printing port

- TSPL function invoking example

- Description: When use TSPL model printing, need to call ageSetupTSPL method tob build a printing buffer in the printer, then call methods like DrawTextTSPL after this will write the data into printer buffer, but will not print immediately, only print after received the print command (TL21 model call PrintTSPL21 method, TL51 model call PrintTSPL51 method), or will not print.

//take USB interface printer as example

//invoking procedure:open port→send printing content→close port；

…

LONG  m_hPrinter;//define port return handle

LONG  ret;//define port print function return value variable

…

//invoke POS_Port_OpenA to open print port

m_hPrinter = POS_Port_OpenA(“SP-USB1”, 1002, false,null);

//identify opening port successfully or not

if(m_hPrinter < 0)

{

POS_Port_Close(m_hPrinter);

MessageBox(_T("open failed"));

return;

}

else

{

MessageBox(_T("open successfully"));

return;

}

…

CString info;

SetIs21();//If TL21 model, need to invoke this method,to confirm the model is TL21, for convenience to distinguish command and related condition.

ret = ClearBuffTSPL(m_hPrinter);//Clear the buffer in printer

if(ret < 0)

{

info.Format("ClearBuffTSPL Parameter,m_hPrinter error:%d",ret);

MessageBox(info);

}

//Set page width, page height, equal to set buffer in the internal printer

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPL Parameter m_hPrinter Error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPL Parameter PageWidth or PageHeight under or equal,error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPL Parameter PageWidth or PageHeight above spefified range, error code:%d",ret);

MessageBox(info);

break;

}

//Send text content to previous internal buffer in printer invoked by PageSetupTSPL,

but will not print immediately

ret = DrawTextTSPL(m_hPrinter,0,0,TRUE,1,1,0,"DrawTextTSPL");

switch(ret)

{

case TSPL_IDERROR:

info.Format("DrawTextTSPL Parameter m_hPrinter Error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("DrawTextTSPL Parameter start_x < 0 or start_y < 0 or xMultiplication < 1 or yMultiplication < 1 or content == NULL,error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("DrawTextTSPL Parameter xMultiplication or yMultiplication above error code:%d",ret);

MessageBox(info);

break;

}

…

PrintTSPL21(m_hPrinter,1);//If TL21 invoked this method, only invoked this method will the printer carry out printing action.

POS_Port_Close(m_hPrinter); //close printing window

### 2.8 POS Function illustration

### LONG  POS_Port_OpenA/W(LPSTR szName, INT iPort, BOOL bFile, LPSTR szFilePath)

Function

Open port

Parameter

- szName:Port Name, For example

Serial port:

No flow control "COM1:9600,N,8,1",

RTS/CTS flow control "COM1:9600,N,8,1:R",

DSR/DTR flow control "COM1:9600,N,8,1:D",

xon/xoff flow control "COM1:9600,N,8,1:X",

Parallel port: "LPT1",

USB: "SP-USB001",

network port: "192.168.1.114:9001";

- iPort: Port Type, 1000 is serial port, 1001 is parallel port, 1002 is USB port, 1003is Ethernet port；

- bFile: choose Whether to save the printing content as the local document

- szFilePath: when bFile is the really numerical value, it is the path of local document saving, otherwise this parameter will be ignored.

Return Numerical Value

- True: Printer handle；

- False: POS_ES_INVALIDPARA(-1)

POS_ES_OVERTIME(-5)

POS_ES_FILEOPENERROR(-6)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Port_Close(LONG iPrinterID)

Function

Close Port

Parameter

- IPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

Return Numerical Value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Output_PrintFontStringA/W(LONG iPrinterID, INT iFont, INT iThick, INT iWidth, INT iHeight, INT iUnderLine, LPCSTR lpString)

Function

Printing formatted characters string

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iFont:when it is 0, printer choose standard ASCII Font A (12 × 24); when it is 1, printer choose compressed ASCII Font B (9 × 17)；

- iThick:when it is 0, printer cancel over-striking mode; when it is 1, printer is over-striking mode

- iWidth:when it is 0, printer cancel the double width mode; when it is 1, printer is double width mode

- iHeight:when it is 0, printer cancel the double height mode; when it is 1, printer is double height mode

- iUnderLine:when it is 0, printer cancel the underline mode; when it is 1, printer is underline mode

- lpString:Character string ended with empty byte

Return Value:

- True: POS_ES_SUCCESS (0)

- False:  POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- Over striking mode is available for character and Chinese character, other mode are only available for character except over striking mode

- when choose twice width and twice height mode at the same time, the character will be magnified twice horizontally and vertically

- the content less then one row should end with “\r\n”to be printed

Printing Effect

POS_Output_PrintFontStringA(m_hPrinter,0,1,1,1,0, “Hi, thank you for choosing our printer, We will get your the best experience!\r\n”);// Printing character string in iThick, iHeight, iWidth mode

### LONG  POS_Control_SetRotaryPrint(LONG iPrinterID,INT n)

Function

Choose/Cancel 90° rotation printing mode

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

- n: when it is 0, printer cancel rotate 90 degree clockwise mode; when it is 1, printer choose rotate 90 degree clockwise mode

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- 90° rotation printing mode is effective for character and Chinese charater, while is noneffective for 1D barcode, 2D barcode and bitmap.

- This function doesn’t support POS76 series printer

Printing Effect

POS_Control_SetRotaryPrint(m_hPrinter,1);// choose 90° rotation printing mode

### LONG  POS_Control_SetInvertedPrint(LONG iPrinterID,INT n)

Function

Choose/Cancel inversion printing mode

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- n:when it is 0, printer cancel inversion printing mode; when it is 1, printer choose inversion printing mode

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- inversion printing mode is effective for character, Chinese character, 1D code, 2D code and bitmap.

Printing Effect

- POS_Control_SetInvertedPrint(m_hPrinter,1);//choose inversion printing mode

### LONG  POS_Control_OppositeColor(LONG iPrinterID, BOOL bOppsite)

Function

Choose/cancel reverse printing mode

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

- bOppsite:0 means cancel reverse printing, 1 means choose reverse printing

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- Reverse printing mode is available for all character( except barcode character)

- Reverse printing mode in precedence to underline mode.Underline mode is ineffective in the reverse printing mode, the set underline will be effective after canceling the reverse printing mode

- Reverse printing mode hasn’t influence on the space between the lines.

- The function don’t support POS76 series printers

Printing Effect

POS_Control_OppositeColor(m_hPrinter,TRUE);//set reverse white printing mode

### LONG  POS_Control_AlignType(LONG iPrinterID, LONG iAlignType)

Function

Setting character alignment method( left alignment/middle alignment/right alignment)

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iAlignType:Method of alignment, “0” Left alignment “1” Middle alignment “2” Right alignment

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- If have set printing area, then align in the printing area

Printing Effect

### LONG  POS_Output_PrintOneDimensionalBarcodeA/W(LONG iPrinterID, INT iType, INT iWidth, INT iHeight, INT hri, LPCSTR lpString)

Function

Printing 1D barcode

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iType:choose one dimensional barcode type. The types which can be chosen refer to constant definition list

- iWidth:choose barcode width，2 ≤iWidth≤ 6；

- iHeight:choose barcode height，1 ≤iHeight≤ 255；

- hri:choose the printing position of barcode character, which can be chosen are below

- POS_HT_NONE:not print；

- POS_HT_DOWN:print under the barcode

- POS_HT_UP:print above the barcode

- POS_HT_BOTH:print both under the barcode and above the barcode

- lpString:barcode data, please refer to the illustration of requirement for barcode data

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- When choose POS_BT_UPCA、POS_BT_UPCE、POS_BT_JAN13、POS_BT_JAN8、POS_BT_CODE39、POS_BT_ITF、POS_BT_CODABAR，barcode data should end with “0”

- This function doesn’t support POS76 series printers

- Barcode data requirement

| Barcode type | Number of amount(n) | Data (d) |
| --- | --- | --- |
| POS_BT_UPCA | 11 ≤ n ≤ 12 | 48 ≤ d ≤ 57 |
| POS_BT_UPCE | 11 ≤ n ≤ 12 | 48 ≤ d ≤ 57 |
| POS_BT_JAN13 | 12 ≤n ≤ 13 | 48 ≤ d ≤ 57 |
| POS_BT_JAN8 | 7 ≤n ≤ 8 | 48 ≤ d ≤ 57 |
| POS_BT_CODE39 | 1 ≤ n ≤ 255 | 45 ≤ d ≤ 57, 65 ≤ d ≤ 90, 32, 36, 37,43 |
| POS_BT_ITF | 1 ≤ n≤ 255 | 48 ≤ d ≤ 57 |
| POS_BT_CODABAR | 1 ≤ n≤ 255 | 48 ≤ d ≤ 57 65 ≤ d ≤ 68, 36, 43,45,46,47 58 |
| POS_BT_CODE93 | 1 ≤ n≤ 255 | 0 ≤ d ≤ 127 |
| POS_BT_CODE128 | 2 ≤ n≤ 255 | 0 ≤ d ≤ 127 |

Printing Effect

POS_Output_PrintOneDimensionalBarcodeA(m_hPrinter,POS_BT_CODE128,2,50,4013,“NO.123456”);//Printing CODE128 barcode

### LONG  POS_Output_PrintTwoDimensionalBarcodeA/W(LONG iPrinterID, INT iType, INT parameter1, INT parameter2, INT parameter3, LPCSTR lpString)

Function

Printing 2D barcode

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iType:choose two dimensional barcode type, two dimensional barcode types which can be chosen refer to constant definition list

- parameter1:

- POS_BT_PDF417:means the character number of each row. The max value of v should less than the max value of model permit because different modes have different paper width

- POS_BT_DATAMATRIX:means the height of figure

- POS_BT_QRCODE:means the version number of figure

- parameter2:

- POS_BT_PDF417:means the class of error correction

- POS_BT_DATAMATRIX:means the width of figure

- POS_BT_QRCODE:means the class of error correction (L:7%, M:15%,Q:25%,H:30%)；

- parameter3:means the magnification times in lengthways，1 ≤parameter3 ≤ 6；

- lpString:barcode data, please refer to the illustration of requirement for barcode data

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- This function doesn’t support POS76 series printers

- Barcode data requirement

| Barcode type | Data length(n) | parameter1(v) | parameter2(r) | parameter3(k) | Data content(d) |
| --- | --- | --- | --- | --- | --- |
| QR Code | 1 ≤n ≤ 65535 | 1 ≤v≤ 16 | r=76,77,81,72 | 1 ≤k≤ 6 | 0 ≤d≤ 255 |
| Data Matrix | 1 ≤n ≤ 65535 | 0 ≤v≤ 144 | 8 ≤r≤ 144 | 1 ≤k≤ 6 | 0 ≤d≤ 255 |
| PDF417 | 1 ≤n ≤ 65535 | 1 ≤v≤ 30 | 0 ≤r≤ 8 | 1 ≤k≤ 6 | 0 ≤d≤ 255 |

Printing Effect

POS_Output_PrintTwoDimensionalBarcodeA(m_hPrinter,POS_BT_PDF417,2,6,6,“www.test.com”);//Print  PDF417 barcode

### LONG  POS_Output_SendLocalFileA/W(LONG iPrinterID, LPCSTR lpFilePath)

Function

Print local TXT document

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- lpFilePath: local TXT document store path

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_FILEOPENERROR(-6)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Output_DownloadRamBmpA/W(LONG iPrinterID, LPCSTR lpFilePath)

Function

Download local monochrome bitmap to printer RAM

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- lpFilePath: Local single color bitmap storage path

Returned value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_NONMONOCHROMEBITMAP(-4)

POS_ES_OVERTIME(-5)

POS_ES_FILEOPENERROR(-6)

POS_ES_OTHERERRORS(-100)

Illustration

- The size of downloaded pic should not beyond the printer RAM area, printer will exit directly if oversize and the data will be regarded as ordinary data.

- The horizontal width of pic should less than the max printable width, the width of 80mm paper is 576dots, the width of 58mm paper is 384dots

- This function can download pic (such as LOGO, digital signature) into the printer RAM before printing, Invoking POS_Output_PrintRamBmp function for printing, it can reduce the waiting time of data transmission when printing pics.

- This function doesn’t support POS76 series printers and POS58 series printers

### LONG  POS_Output_PrintRamBmp(LONG iPrinterID,INT n)

Function

Print local monochrome bitmap downloaded to printer RAM according to the chosen mode

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- n:choose bitmap printing mode，0 ≤ n ≤ 3；

| n | mode | - vertical resolution (DPI) | - lateral resolution (DPI) |
| --- | --- | --- | --- |
| 0 | normal | 203 | 203 |
| 1 | Twice width | 203 | 101 |
| 2 | Twice height | 101 | 203 |
| 3 | Twice width&Twice height | 101 | 101 |

Illustration

- This function doesn’t support POS76 series printers and POS58 series printers

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Output_PrintBmpDirectA/W(LONG iPrinterID, LPCSTR lpFilePath)

Function

Print local single color bitmap

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- lpFilePath: local monochrome bitmap store path

Illustration

This function doesn’t support POS76 series printers

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_NONMONOCHROMEBITMAP(-4)

POS_ES_OVERTIME(-5)

POS_ES_FILEOPENERROR(-6)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Output_PrintBmpDirectA_POS76(LONG iPrinterID, LPCSTR lpFilePath)

Function

Dot matrix type printing local single color bitmap

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- lpFilePath: local monochrome bitmap store path

Illustration

This function support POS76 series dot matrix printers, not support thermal printers.

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_NONMONOCHROMEBITMAP(-4)

POS_ES_OVERTIME(-5)

POS_ES_FILEOPENERROR(-6)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Output_PrintStringA/W(LONG iPrinterID, LPSTR lpString)

Function

Print characters string

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- lpString: characters string which end with the empty byte

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- the content less than one row should end with “\r\n”to be printed

### LONG  POS_Output_PrintFlashBmp(LONG iPrinterID,INT n)

Function

Print pr-downloaded bitmap

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- n:bitmap store number，1 ≤ n ≤ 8；

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- This function should be used with printer setting tool, firstly download bitmap in specific position by setting tool, then use the function to print

### LONG  POS_Control_PrintTestpage(LONG iPrinterID)

Function

Print test page

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Print Effect

### LONG  POS_Output_PrintData(LONG iPrinterID, LPCSTR lpBuffer, INT iLength)

Function

Send hexadecimal data to printer

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- lpBuffer:The date will be sent in the buffer;

- iLength:the size of buffer

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Invoking example

POS_Output_PrintStringA(m_hPrinter, “Hello world!\r\n”);Print“Hello world!”

byte[] cmd1 = { 0x48,0x65,0x6C,0x6C,0x6F,0x20,0x77,0x6F,0x72,0x6C,0x64,0x21, 0x0A};

POS_Output_PrintData(m_hPrinter, cmd1, 13);Print“Hello world!”

…

byte[] cmd2 = { 0x1d, 0x56, 0x42, 0x00 };//cutting paper instruction

LONG ret = POS_Output_PrintData(m_hPrinter, cmd2, 4);//Sent hex data

//identify sending printing content successfully or not

switch (ret)

{

case POS_ES_SUCCESS:

MessageBox(_T("sending successfully"));

break;

case POS_ES_INVALIDPARA:

MessageBox(_T("Parameter error"));

break;

case POS_ES_WRITEFAIL:

MessageBox(_T("sending failed"));

break;

case POS_ES_OVERTIME:

MessageBox(_T("overtime"));

break;

case POS_ES_OTHERERRORS:

MessageBox(_T("other error"));

break;

}

…

### LONG  POS_Control_SetPrintPosition(LONG iPrinterID, LONG iLeftMargin, LONG iWidth)

Function

Set the printing left margin and printing width

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- iLeftMargin:left margin, default is “0”

- iWidth:set printing area, default is “0”

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- Use iLeftMargin to set left margin, iLeftMargin is the left margin printing dots which need to be set

- Use iWidth to set printing area, iWidth is the printing dots which need to be set in printing area. If [left margin+printing width] is over the printable area, the printing area = the printable area -left margin

- the max printing area of 80mm paper is 576 dots, the max printing area of 58mm paper is 384 dots, the max printing area of 76mm paper is 420dots,

- This function doesn’t support POS76 series printers and POS58 series printers

Print Effect

POS_Control_SetPrintPosition(m_hPrinter, 80, 80);//set print left margin to 80dots, then printable width is 80dots

### LONG  POS_Control_SetLineSpace(LONG iPrinterID, INT iSpace)

Function

Setting printing height of line(the height of character+line space)

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- iSpace:set height of line，0 ≤ n ≤ 255；

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Print Effect

POS_Control_SetLineSpace(m_hPrinter, 80);//set the height of line to 80 dots

### LONG POS_Control_SetPrintFontE(LONG iPrinterID,BOOL iFont,BOOL iBold,BOOL iDoubleWidth,BOOL iDoubleHeight, BOOL  iUnderLine)

Function

Set the print format of western characters font

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- iFont: Font: true=Normal font(12x24), false=Compressed font(9x17);

- iBold: Over striking; true=over striking, false=normal;

- iDoubleWidth: Twice width: true=twice width, false=normal；

- iDoubleHeigh: Twice height: true=twice height, false=normal；

- iUnderLine: Underline: true=underline，false=non-underline；

Return Numerical Value

- Ture:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Print effect

POS_Control_SetPrintFontE(m_hPrinter,true,true,true,true,true);//Set the print format of western character font

### LONG POS_Control_SetPrintFontC(LONG iPrinterID,BOOL iDoubleWidth,BOOL iDoubleHeight,BOOL iUnderLine)

Function

Set the print format of Chinese character font

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port；

- iFont:Font:true=Normal font(12x24)，false=Compressed font(9x17)；

- iDoubleWidth:Twice width:true=twice width，false=normal；

- iDoubleHeight:Twice height:true=twice height，false=normal；

- iUnderLine:Underline:true=underline，false=non-underline；

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Print Effect

POS_Control_SetPrintFontC(m_hPrinter, true, true, true);//Set the print format of Chinese character font

### LONG  POS_Control_CutPaper(LONG iPrinterID, LONG iType, LONG iLines)

Function

Content of printing buffer, feeding paper line which set by ilines and cutting paper

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iType:Cutter type, 1 “Full Cut”  0  “Partial Cut”

- iLines:The line of paper feeding；

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- full cut/partial cut need the support of cutter

- This function doesn’t support POS58 series printer

### LONG  POS_Control_FeedLines(LONG iPrinterID, LONG iLines)

Function

feeding paper line which is set by parameter ilines

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iLines: The line of feeding paper forward, 0 ≤iLines≤ 255；

Return Numerical Value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Control_CashDraw(LONG iPrinterID, LONG iPort, LONG iTime1, LONG iTime2)

Function

Open cash drawer

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iPort: Cash drawer number:  1 means to open 1# cash drawer, 2 means to open 2# cash drawer, 3 means to open cash drawer 1# 2# cash drawers.

- iTime1: Time of open cash drawer  [iTime1 × 2 ms], 0 ≤iTime1≤ 255;

- iTime2: Time of close cash drawer  [iTime2× 2 ms], 0 ≤iTime2≤ 255;

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- the time of opening cash drawer should be determined by the used cash draw parameter

### LONG  POS_Control_BlackMark(LONG iPrinterID)

Function

Feeding paper to black mark position

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

Return Numerical Value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

### LONG  POS_Status_RTQueryStatus(LONG iPrinterID)

Function

Checking status of printer paper

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

Return Numerical Value

- True:  POS_ES_SUCCESS(0)

POS_ES_PAPEROUT(1)

- False:  POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- This function is to check status of printer paper, paper in return POS_ES_SUCCESS (0), paper out return POS_ES_PAPEROUT(1)

Invoking example

…

LONG ret = POS_Status_RTQueryStatus(m_hPrinter);

switch (ret)

{

case POS_ES_SUCCESS:

MessageBox(_T("paper in"));

break;

case POS_ES_PAPEROUT:

MessageBox(_T("paper out"));

break;

case POS_ES_INVALIDPARA:

MessageBox(_T("Parameter error"));

break;

case POS_ES_WRITEFAIL:

MessageBox(_T("writing failed"));

break;

case POS_ES_READFAIL:

MessageBox(_T("reading failed"));

break;

case POS_ES_OVERTIME:

MessageBox(_T("overtime"));

break;

case POS_ES_OTHERERRORS:

MessageBox(_T("other error"));

break;

}

…

### LONG  POS_Status_RTQueryTypeStatus(LONG iPrinterID,INT n)

Function

Check status of printer status

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

- n: check type number, 1、2、3、4;

Return Numerical Value

- True:POS_ES_SUCCESS(0)

POS_ES_PAPEROUT(1)

POS_ES_DOOROPEN(3)

POS_ES_CUT(4)

POS_ES_DRAWERHIGH(5)

POS_ES_PAPERENDING(6)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- Parameter n means: n=1, printer status; n=2, off-line status; n=3，error status; n=4, paper sensor status;

Invoking example

…

//take parameter n=2, check printer off-line status as example

int nDoorOpen = 0;   //paper case open

LONG ret = POS_Status_RTQueryTypeStatus(m_hPrinter,2);

switch (ret)

{

case POS_ES_INVALIDPARA:

MessageBox(_T("Parameter error"));

return;

break;

case POS_ES_WRITEFAIL:

MessageBox(_T("writing failed"));

return;

break;

case POS_ES_READFAIL:

MessageBox(_T("Reading failed"));

return;

break;

case POS_ES_OVERTIME:

MessageBox(_T("over time"));

return;

break;

case POS_ES_OTHERERRORS:

MessageBox(_T("other error));

return;

break;

}

if (ret == POS_ES_DOOROPEN)

{

nDoorOpen = 1;

}

CString strMessage;

strMessage = _T("#");

if (nDoorOpen == 1)

{

strMessage = strMessage + _T("paper case open") + _T("#");

}

else

{

strMessage = strMessage + _T("paper case close") + _T("#");

}

MessageBox(strMessage);

…

### LONG  POS_Status_QueryTaskStatus(LONG iPrinterID,INT second)

Function

Check the printing task finished or not

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- Second: it is unit second, that is over time value set by users, which should be more than the time of printing task; if the time of printing task is more than the time function set by user, which lead to printing task unfinished, at this time, if the setting time value is less than 5seconds, printer will regard it as 5 seconds.

Return Numerical Value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- The function is to check the printer execute this command or not, that is to check the print task before this command is finished or not.

- This function doesn’t support POS76 series printers

Invoking example

…

Send printing task

…

//overtime valuesecond为5s

LONG ret = POS_Status_QueryTaskStatus(m_hPrinter,5);

switch (ret)

{

case POS_ES_SUCCESS:

MessageBox(_T("printing finished"));

break;

case POS_ES_READFAIL:

MessageBox(_T("reading data error"));

break;

case POS_ES_INVALIDPARA:

MessageBox(_T("parameter error"));

break;

case POS_ES_WRITEFAIL:

MessageBox(_T("sending failed"));

break;

case POS_ES_OVERTIME:

MessageBox(_T("task printing…"));

break;

case POS_ES_OTHERERRORS:

MessageBox(_T("other error"));

break;

}

…

### LONG  POS_Input_PrinterId(LONG iPrinterID, char *buf)

Function

Get printer factory information

Parameter

- iPrinterID: Printer handle, it will be confirmed by the return Numerical Value from the opened port

- char *buf: receiving data buf defined by user

Return Numerical Value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- The function returned factory information is “_” + “factory name”

### LONG  POS_Control_ReSet(LONG iPrinterID)

Function

Initialize the printer

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

Return Numerical Value

- True: POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

POS_ES_WRITEFAIL(-2)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Illustration

- The function is to initialize the printer, and to clear the data of printing buffer. The printer mode will be set in the default mode when power on.

### LONG  Color24_GrayBW(LPCTSTR szSourceFile,LPCTSTR szTargetFile)

Function

To convert the 24bit bitmap into monochrome bitmap

Parameter

- szSourceFile: The store path of bitmap needed to be converted

- szTargetFile: The store path of converted bitmap

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

POS_ES_READFAIL(-3)

POS_ES_FILEOPENERROR (-6)

POS_ES_OTHERERRORS(-100)

### VOID  ASBCallback(int status, void * param)

Function

User-provided callback function

Parameter

- status: Save the status reported by the printer.

- Param: dialog pointer

Return Numerical Value

- None

### LONG  POS_Status_ASB(LONG iPrinterID, INT n, ASBCallBack p, void * pDlgOwer)

Function

Set automatic status return

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- n:0: Disable automatic status return. 1: Turn on automatic status return.

- p: callback function pointer

- pDlgOwer: dialog pointer

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

- POS_ES_WRITEFAIL(-2)

- POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Call Instance

void ASBCallBack(int ASBStatus[POS_ASB_STATUS_NUM],void* pDlgOwner)

{

CString strMessage;

strMessage = _T("#");

if (ASBStatus[POS_Drawer_Heigh] == 1)

{

strMessage = strMessage + _T("Cash drawer Vih") + _T("#");

}

else

{

strMessage = strMessage + _T("Cash drawer Vil") + _T("#");

}

if (ASBStatus[POS_Offline] == 1)

{

strMessage = strMessage + _T("Printer offline") + _T("#");

}

else

{

strMessage = strMessage + _T("Printer online") + _T("#");

}

if (ASBStatus[POS_Door_Open] == 1)

{

strMessage = strMessage + _T("Cover open") + _T("#");

}

else

{

strMessage = strMessage + _T("Cover closed") + _T("#");

}

if (ASBStatus[POS_Feed] == 1)

{

strMessage = strMessage + _T("Feeding paper with the feed button") + _T("#");

}

else

{

strMessage = strMessage + _T("Paper is not fed with the feed button") + _T("#");

}

if (ASBStatus[POS_Cut] == 1)

{

strMessage = strMessage + _T("Cutter errors") + _T("#");

}

else

{

strMessage = strMessage + _T("No cutter errors") + _T("#");

}

if (ASBStatus[POS_Unrecoverable_Error] == 1)

{

strMessage = strMessage + _T("Unrecoverable errors") + _T("#");

}

else

{

strMessage = strMessage + _T("No unrecoverable errors") + _T("#");

}

if (ASBStatus[POS_AutoRecoverable_Error] == 1)

{

strMessage = strMessage + _T("Auto-recovery errors") + _T("#");

}

else

{

strMessage = strMessage + _T("No auto-recovery errors") + _T("#");

}

if (ASBStatus[POS_PaperEnding] == 1)

{

strMessage = strMessage + _T("Paper will run out") + _T("#");

}

else

{

strMessage = strMessage + _T("Paper is enough") + _T("#");

}

if (ASBStatus[POS_PaperOut] == 1)

{

strMessage = strMessage + _T("Out of paper") + _T("#");

}

else

{

strMessage = strMessage + _T("Paper available") + _T("#");

}

if (pDlgOwner)

{

CDialogESPOS* p = (CDialogESPOS *)pDlgOwner;

p->m_ASBStatus.SetWindowText(strMessage);

if (m_hPrinter == 0)

{

CString prompt;

p->m_ASBButton.GetWindowText(prompt);

if (prompt == IDS_ASB_Close)

{

prompt.LoadString(IDS_ASB_Open);

p->m_ASBButton.SetWindowText(prompt);

p->MessageBox("Close ASB");

}

}

}

return;

}

.....

LONG ret = POS_Status_ASB(m_hPrinter,1, ASBCallBack, (void *)this);

switch (ret)

{

case POS_ES_INVALIDPARA:

MessageBox(_T("Parameter error"));

break;

case POS_ES_WRITEFAIL:

MessageBox(_T("Failed to send"));

break;

case POS_ES_OVERTIME:

MessageBox(_T("Timeout"));

break;

case POS_ES_OTHERERRORS:

MessageBox(_T("Other errors"));

break;

}

### LONG POS_Control_SetFontSize (LONG iPrinterID,  INT iWidthSize,INT iHeightSize)

Function

Set character size

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- iWidthSize: Horizontal magnification.

- iHeightSize: Vertical magnification.

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False:POS_ES_INVALIDPARA(-1)

- POS_ES_WRITEFAIL(-2)

- POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Print Result

POS_Control_SetFontSize(m_hPrinter,2,2); //The length and width of the font are enlarged twice

POS_Output_PrintStringA(m_hPrinter,”Hi, thank you for choosing our printer,We will get your the best experience!\r\n”);

### LONG  POS_Control_SetTabs (LONG iPrinterID ,LPCSTR pszPosition,INT count)

Function

Set the tab position

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

- pszPosition: This parameter points to the tab position of the buffer storage, terminated by 0.

- count: Set the number of tab positions, 0 <= nCount <= 32.

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

- POS_ES_WRITEFAIL(-2)

- POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

### LONG POS_Control_ExecuteTabs (LONG iPrinterID)

Function

Execute tab

Parameter

- iPrinterID:Printer handle, it will be confirmed by the return Numerical Value from the opened port

Return Numerical Value

- True:POS_ES_SUCCESS (0)

- False: POS_ES_INVALIDPARA(-1)

- POS_ES_WRITEFAIL(-2)

- POS_ES_READFAIL(-3)

POS_ES_OVERTIME(-5)

POS_ES_OTHERERRORS(-100)

Print Result

....

//Normal printing

POS_Output_PrintFontStringA(m_hPrinter,0,0,0,0,0,"菜品名称        数量      规格       小计\r\n");

POS_Output_PrintFontStringA(m_hPrinter,0,0,0,0,0,"——————————————————————\r\n");

POS_Output_PrintFontStringA(m_hPrinter,0,0,0,0,0,"健怡雪碧        4            瓶      40.00\r\n");

POS_Output_PrintFontStringA(m_hPrinter,0,0,0,0,0,"鲟龙鱼        1.5            斤      28.00\r\n");

//Tabbed printing

char position[] = {0x01,0x0f,0x1e,0x27,0x00};   //指定每格的位置

POS_Control_SetTabs(m_hPrinter,position,4);

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"黑鱼");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"2.0");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"斤");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"30.00\r\n");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"烤鸭");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"1.0");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"只");

POS_Control_ExecuteTabs(m_hPrinter);

POS_Output_PrintStringA(m_hPrinter,"300.00\r\n");

char end = 0x00;

POS_Control_SetTabs(m_hPrinter,&end,0); //结束跳格

## 2.9  TSPL Function illustration

### LONG  SetIs21();

Function

To confirm the model is TL21, to distinguish internal command and related condition

Parameter

- None

Returned value

- True: TSPL_SUCCESS(0)

- False: None(no error returned value for the method)

Invoked example

SetIs21();//If it is TL21, printer will need this method

### LONG  PageSetupTSPL(LONG iPrinterID, INT PageWidth, INT PageHeight);

Function

Set page width and page height, build buffer in internal printer

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- PageWidth: Page width

- PageHeight: Page height

Returned value

- True: TSPL_SUCCESS(0)

- False: TSPL_IDERROR(-1): iPrinterID Return this error when got error

TSPL_PARAM_LESS_EQUAL_ZERO(-2): When using TL21, return this error when ParameterPageWidth < 1 or PageHeight < 1. When using TL51, return this error when ParameterPageWidth < 1 or PageHeight < 1.

- TSPL_PARAM_GREAT_RANGE (-3):  When using TL21, return this error when ParameterPageWidth > 45 or PageHeight > 56. When using TL51, return this error when ParameterPageWidth > 100or PageHeight > 225.

- Others: TL51printer max label square is length*width= 225X100mm

- TL21printer max label square is length*width= 56X45mm

Invoke example

LONG ret =0；

CString info;

SetIs21();//If TL21, the printer will need this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPL error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight under or equal 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight above specified range, error code:%d",ret);

MessageBox(info);

break;

}

### LONG  DrawLineTSPL(LONG iPrinterID,INT StartX, INT StartY, INT LineWidth, INT LineHeight);

Function

Draw line

Parameter

- iPrinterID: Printer handle, is decided by returned value which open port

- startX: Start at left above level direction, in dots(cannot exceed page width)

- startY: Start at left top corner in vertical direction, in dots(cannot exceed page height)

- lineHeight: Line height, in dots(cannot exceed page height)

- lineLength: Line width, in dots(cannot exceed page width)

- Returned value

- True: TSPL_SUCCESS(0)

- False: TSPL_IDERROR(-1): iPrinterID Return this error when error occur

- TSPL_PARAM_LESS_EQUAL_ZERO(-2): Parameter StartX under 0 or StartY under 0 or LineWidth under 0 or LineHeight under 0 return this error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 need this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth或PageHeight under or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight above or equal to zero，error code:%d",ret);

MessageBox(info);

break;

}

ret = DrawLineTSPL(m_hPrinter,10,10,100,100);

switch(ret)

{

case TSPL_IDERROR::

info.Format("DrawLineTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("DrawLineTSPLParameter StartX under 0 or StartY under 0 or LineWidth under 0 or LineHeight under 0 return this error，error code:%d",ret);

MessageBox(info);

break;

}

PrintTSPL21(m_hPrinter,1);

//PrintTSPL(m_hPrinter,1,1);//TL51 need this method

### LONG  PrintTSPL21(LONG iPrinterID,INT Set);

Function

- Only received PRINT order will the printer carry out printing action, otherwise do not print

Parameter

- iPrinterID: Printer handle, is decided by returned value which opened the port

- Set: Specify the printing pieces(set)

- Returned value

- True: TSPL_SUCCESS(0)

- Error: TSPL_IDERROR(-1): Parameter iPrinterID returned this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2): Parameter Set returned this value when the value is under 1

- TSPL_PARAM_GREAT_RANGE(-3): Parameter Set above 65535

说明

- TL21 model printing method

### LONG  PrintTSPL51(LONG iPrinterID,INT Set,INT Copy);

Function

- Only received PRINT order will the printer carry out printing action, otherwise do not print

Parameter

- iPrinterID: Printer handle, is decided by returned value which opened the port

- Set:Specify the printing pieces(set)

- Copy: Print how many pieces for every barcode page on every piece

- Returned value

- True:  TSPL_SUCCESS(0)

- Error:  TSPL_IDERROR(-1): ParameteriPrinterID return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):   ParameterSet or Copy under 1 return this value

- TSPL_PARAM_GREAT_RANGE(-3):   ParameterSet or Copy above 65535 return this value

Description

- TL51 model printing method

### LONG  DrawBorderTSPL(LONG iPrinterID,INT LineWidth, INT top_left_x, INT top_left_y, INT bottom_right_x, INT bottom_right_y);

Function

Draw frame

Parameter

- iPrinterID: printer handle, is decided by Returned value of opened port

- lineWidth: rectangle line width, in dots (cannot exceed label width)

- top_left_x: the start position x on up left corner on horizontal direction of rectangle, in dots(cannot exceed page width)

- top_left_y: start position y at up left corner in vertical direction of rectangle, in dots(cannot exceed page height)

- bottom_right_x: the end position x on right bottom corner on horizontal direction of rectangle, in dots(cannot exceed page width)

- bottom_right_y: the end position y on right bottom corner on vertical direction of rectangle, in dots(cannot exceed page height)

Returned value

- True:  TSPL_SUCCESS(0)

- False:  TSPL_IDERROR(-1): ParameteriPrinterID return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):  当ParameterLineWidth < 0 or top_left_x < 0 or top_left_y < 0 or bottom_right_x < 0 or bottom_right_y < 0, return this error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 need this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR::

info.Format("PageSetupTSPL error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth或PageHeight under or equal to小于或0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight above specified range, error code:%d",ret);

MessageBox(info);

break;

}

ret = DrawBorderTSPL(m_hPrinter,5,20,20,200,200);

switch(ret)

{

case TSPL_IDERROR::

info.Format("DrawBorderTSPL error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("DrawBorderTSPLParameterLineWidth < 1 or top_left_x < 1 or top_left_y < 1 or bottom_right_x < 1 or bottom_right_y < 1，error code:%d",ret);

MessageBox(info);

break;

}

PrintTSPL21(m_hPrinter,1);

//PrintTSPL(m_hPrinter,1,1);//TL51 printer need this method

### LONG DrawTextTSPL(LONG iPrinterID,INT start_x, INT start_y, BOOL isSimplifiedChinese, INT xMultiplication,INT yMultiplication, INT rotate, CString content);

Function

Print text

Parameter

- iPrinterID: printer handle, is decided by returned value of opened port.

- start_x: Starting position coordinate of words on x direction, in dots(cannot exceed page width)

- start_y:Starting position coordinate of words on y direction,(cannot exceed page height)

- isSimplifiedChinese: true simplified Chinese24×Font(GB Code);false traditional Chinese 24×Font(BIG 5)

- xMultiplication: Magnification times range on X direction  1—4

- yMultiplication: Magnification times range on Y direction 1—4

- rotate: rotate angle of text, clockwise is 0, not rotate 90, clockwise 180, clockwise 270, value is 0,90,180,270

- content: The content which is going to be printed, please use \[“] to print double quotes mark in the program.

- Please use \[R] to print CR in program if print D(hex) characters	            Please use \[A] to print LF in program if print D(hex) characters

Returned value

- True: TSPL_SUCCESS(0)

- Error: TSPL_IDERROR(-1): ParameteriPrinterID , return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2): when Parameterstart_x < 0 or start_y < 0 or xMultiplication < 1 or yMultiplication < 1 or content == NULL, return this error

TSPL_PARAM_GREAT_RANGE(-3): ParameterxMultiplication or yMultiplication is above 4，or rotate Parameter value is among 0,90,180,270, return this error

Invoke example:

LONG ret =0；

CString info;

SetIs21();//TL21 printer need this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth或PageHeight under or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is above specified range, error code，error code:%d",ret);

MessageBox(info);

break;

}

ret = DrawTextTSPL(m_hPrinter,0,0,TRUE,1,1,0,"DrawTextTSPL");

switch(ret)

{

case TSPL_IDERROR:

info.Format("DrawTextTSPLParameterm_hPrinter error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

- info.Format("DrawTextTSPL Parameterstart_x < 0 or start_y < 0 or xMultiplication < 1 or yMultiplication < 1 or content == NULL,error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("DrawTextTSPL ParameterxMultiplication or yMultiplication above 4

Error code:%d",ret);

MessageBox(info);

break;

}

PrintTSPL21(m_hPrinter,1);

//PrintTSPL(m_hPrinter,1,1);//TL51 printer use this method

### LONG DrawBarCodeTSPL(LONG iPrinterID,INT start_x, INT start_y, CString type, INT height, BOOL isReadable,INT rotate, INT narrowWidth, INT wideWidth, CString content);

Function

Print 1D barcode

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- start_x: Upper left corner position on horizontal direction of barcode, in dots(cannot exceed page width)

- start_y: Upper left corner position on vertical direction of barcode, in dots(cannot exceed page height)

- type: barcode type, including “UPCA”，”UPCE”，”EAN13”，”EAN8”，”39”，”CODABAR”，”93”，”128”

- height: barcode hright, in dots, (cannot exceed page height)

- isReadable: FALSE, cannot be seen by human eyes, TRUE，can be seen by human eyes

- rotate: text rotate angle, clockwise is 0, not rotate 90, clockwise 180, clockwise 270, value is 0,90,180,270

- narrowWidth: width of narrow strip of barcode, in dots

- wideWidth: width of wide strip of barcode, in dots

- content: Barcode content, can only be number numeric string, otherwise will print error.

Returned value

- True:  TSPL_SUCCESS(0)

- False:  TSPL_IDERROR(-1): ParameteriPrinterID, returned this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):  start_x < 0 or start_y < 0 or height < 0 or rotate < 0 or narrowWidth < 0 or wideWidth < 0 or content == NULL, return this error

TSPL_PARAM_GREAT_RANGE(-3):  rotate Parameter value is not among 0,90,180,270 or type is not “UPCA”，”UPCE”，”EAN13”，”EAN8”，”39”，”CODABAR”，”93”，”128”, return this error.

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight under or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth或PageHeight above specified range, error code:%d",ret);

MessageBox(info);

break;

}

ret = DrawBarCodeTSPL(m_hPrinter,10,10,"128",30,TRUE,0,2,4,"123456");

switch(ret)

{

case TSPL_IDERROR:

info.Format("DrawBarCodeTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

- info.Format("DrawBarCodeTSPL Parameterstart_x < 0 or start_y < 0 or height < 0 or rotate < 0 or narrowWidth < 0 or wideWidth < 0 orcontent == NULL

error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("DrawBarCodeTSPL ParameterrotateParameter value is not among 0,90,180,270, or type is not “UPCA”，”UPCE”，”EAN13”，”EAN8”，”39”，”CODABAR”，”93”，”128” error code:%d",ret);

MessageBox(info);

break;

}

PrintTSPL21(m_hPrinter,1);

//PrintTSPL(m_hPrinter,1,1);//TL51 printer need to use this method

### LONG  ClearBuffTSPL(LONG iPrinterID);

Function

Clear the printer buffer

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- Returned value

- True:TSPL_SUCCESS(0)

- Error:TSPL_IDERROR(-1): ParameteriPrinterID , return this error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

### LONG  GetPrinterStatusTSPL(LONG iPrinterID)

Function

Gain printer status

Parameter

iPrinterID: Printer handle, is confirmed by returned value of opened port

- Returned value

- True:  TSPL_SUCCESS(0)

- False:  TSPL_IDERROR(-1): ParameteriPrinterID, return this value

TSPL_PRINTER_STATUS_OUTPAPER(1): printer is paper out

TSPL_PRINTER_STATUS_WORK(2): printing

TSPL_PRINTER_STATUS_ENCLOSURENOCLOSE(3): paper house opened

TSPL_PRINTER_STATUS_ERROR(4): printer internal error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need this method

ret = GetPrinterStatusTSPL(m_hPrinter);

switch(ret)

{

case TSPL_IDERROR:

info.Format("GetPrinterStatusTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PRINTER_STATUS_OUTPAPER:

info.Format("GetPrinterStatusTSPL, printer is paper out:%d",ret);

MessageBox(info);

break;

case TSPL_PRINTER_STATUS_WORK:

info.Format("GetPrinterStatusTSPL,printing:%d",ret);

MessageBox(info);

break;

case TSPL_PRINTER_STATUS_ENCLOSURENOCLOSE:

info.Format("GetPrinterStatusTSPL, paper house opened:%d",ret);

MessageBox(info);

break;

case TSPL_PRINTER_STATUS_ERROR:

info.Format("GetPrinterStatusTSPL,printer internal error:%d",ret);

MessageBox(info);

break;

}

### LONG  DriveBeepTSPL(LONG iPrinterID)

Function

Drive the buzzer beep one sound

Parameter

iPrinterID: Printer handle, is confirmed by returned value of opened port

- Returned value

- True:TSPL_SUCCESS(0)

- False:TSPL_IDERROR(-1): ParameteriPrinterID , return this value

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need this method

ret = DriveBeepTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("DriveBeepTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

### LONG  SetPaperbackOrPaperFeedTSPL(LONG iPrinterID,BOOL isFeedBack, INT mDot)

Function

Control paper feeding or back

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- isFeedBack: whether back paper, true back paper

- mDot: 1≤mDot≤1000

- Returned value

- True: TSPL_SUCCESS(0)

- False: TSPL_IDERROR(-1): ParameteriPrinterID, return this error

- TSPL_PARAM_GREAT_RANGE(-3): when ParametermDot less than 1 or more than 1000,

- return this error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need to use this method

ret = SetPaperbackOrPaperFeedTSPL(m_hPrinter,TRUE,50);

switch(ret)

{

case TSPL_IDERROR:

info.Format("SetPaperbackOrPaperFeedTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("SetPaperbackOrPaperFeedTSPL when ParametermDot is under 1 or above1000, return this value:%d",ret);

MessageBox(info);

break;

}

### LONG  ReverseAreaTSPL(LONG iPrinterID,INT start_x, INT start_y, INT width, INT height)

Function

Reverse printing in specified area

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- start_x: Top left corner X coordinate in the area, in dots,(can’t exceed the page width)

- start_y : Top left corner Y coordinate in the area, in dots,(can’t exceed the page height)

- width:area width ,in dots(can’t exceed page width)

- height: area height ,in dots(can’t exceed page height)

- Returned value

- True: TSPL_SUCCESS(0)

- Error: TSPL_IDERROR(-1): ParameteriPrinterID , return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):  start_x < 0 or start_y < 0 or height < 0 or width < 0

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need to use this method

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is under or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is above specified scope，error code:%d",ret);

MessageBox(info);

break;

}

ret = ReverseAreaTSPL(m_hPrinter,10,10,45,56);

switch(ret)

{

case TSPL_IDERROR:

info.Format("ReverseAreaTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

- info.Format("ReverseAreaTSPLParameterstart_x < 0 or start_y < 0 or height < 0 or width < 0，error code:%d",ret);

MessageBox(info);

break;

}

PrintTSPL21(m_hPrinter,1);//TL21 printer need to use this method

//PrintTSPL(m_hPrinter,1,1);//TL51 printer need to use this method

### LONG  SetGAPTSPL(LONG iPrinterID,DOUBLE value)

Function

Set the vertical distance between labels

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- value: 0≦value≤25.4(mm)

- Returned value

- True:TSPL_SUCCESS(0)

- Fault:TSPL_IDERROR(-1): ParameteriPrinterID , return this error

- TSPL_PARAM_GREAT_RANGE(-3):  when Parametervalue is under or equal to 1, or above or equal to 25.4, return this error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 need to use this method

ret = SetGAPTSPL(m_hPrinter,10);

switch(ret)

{

case TSPL_IDERROR:

info.Format("SetGAPTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("SetGAPTSPL when Parametervalue is under or equal to 1, or above or equal to 25.4, return this error error:%d",ret);

MessageBox(info);

break;

}

### LONG  SetLabelReferenceTSPL(LONG iPrinterID,INT x,INT y)

Function

Define original reference coordinate of label

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- X: Coordinate position on horizontal direction, in dots(cannot exceed page width)

- y: Coordinate position on vertical direction, in dots(cannot exceed page height)

- Returned value

- True:TSPL_SUCCESS(0)

- Fault:TSPL_IDERROR(-1): ParameteriPrinterID,  return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2): x < 0 || y < 0 , return this error

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 printer need to use this method

ret = SetLabelReferenceTSPL(m_hPrinter,10,10);

switch(ret)

{

case TSPL_IDERROR:

info.Format("SetLabelReferenceTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

- info.Format("SetLabelReferenceTSPLParameterx < 0 || y < 0 返回此 error，error code:%d",ret);

MessageBox(info);

break;

}

### LONG  DownLoadBitMapTSPL(LONG iPrinterID,BOOL isMoveFlash,CString PathName)

Function

Download bitmap to printer

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- isMoveFlash: false: download bitmap to printer memory(data in printer memory will lost if loose power)；true: download to flash

- PathName: Download to memory or FLASH, path or name, name could contain 8 characters at most.

- Returned value

- True:TSPL_SUCCESS(0)

- Fault:TSPL_IDERROR(-1): ParameteriPrinterID got error or failed to open file, return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2): ParameterPathName==NULL return this error

- Description

- TL21 model does not support this method at present, TL51 support.

Invoke example

LONG ret =0；

CString info;

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,100,225);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is under or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is above specified range，error code:%d",ret);

MessageBox(info);

break;

}

ret = DownLoadBitMapTSPL(m_hPrinter,FALSE,”D://LOG.bmp”);

switch(ret)

{

case TSPL_IDERROR:

info.Format("DownLoadBitMapTSPLParameterm_hPrinter error or failed to open file:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("DownLoadBitMapTSPLParameterPathName==NULL:%d",ret);

MessageBox(info);

break;

}

ret = PutBitMapTSPL(m_hPrinter,10,10,”D://LOG.BMP”);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PutBitMapTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PutBitMapTSPLParameterfileName==NULL:%d",ret);

MessageBox(info);

break;

}

PrintTSPL(m_hPrinter,1,1);

### LONG  Draw2DBarCodeTSPL(LONG iPrinterID,INT itype, INT start_x, INT start_y, INT para1, INT para2, INT para3, LPCSTR content)

Function

Print 2D barcode

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- itype: Barcode type. The types which can be chosen refer to TSPL constant definition list

- start_x: coordinate position on horizontal direction, in dots(cannot exceed page widht)

- start_y: coordinate position on vertical direction, in dots(cannot exceed page height)

- para1: When the barcode type is QRCODE, it is the error correction level, and the values are 7,15,25 and 30, representing the error correction levels of 7%, 15%, 25% and 30% respectively; When the barcode type is PDF417 or DMATRIX, it is the graphic height.

- para2: When the barcode type is QRCODE, it is the cell width, and the value is an integer of 1~10; When the barcode type is PDF417 or DMATRIX, it is the graphic width.

- para3: When the barcode type is DMATRIX, it is the module size, and the value is an integer of 0~5; When the barcode type is PDF417 or QRCODE, it is a counterclockwise rotation angle, and the value is 0,90,180,270, which means 0 degrees, 90 degrees, 180 degrees and 270 degrees respectively.

- content: 2D Barcode content

- Returned value

- True:TSPL_SUCCESS(0)

- Fault:TSPL_IDERROR(-1): Parameter iPrinterID got error, return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):  Parameter start_x < 0 or start_y < 0 or content

- == NULL

- TSPL_PARAM_GREAT_RANGE(-3):  When the barcode type is DATAMATRIX, the value of the

- parameter para3 is not in the range of 0~5, or when the barcode type is QRCODE, the value

- of parameter para2 is not in the range of 1~10. This error is returned when the above two

- conditions occur

- TSPL_PARAM_INVALID(-4): The content length is 0 or when the barcode type is PDF417, the

- value of the parameter para3 is not 0, 90, 180, 270 or when the barcode type is QRCODE, the

- value of the parameter para1 is not 7, 15, 25, 30 or when the barcode type is QRCODE, the

- value of the parameter para3 is not 0, 90, 180, 270. This error will be returned when the above

- four conditions occur.

- Description

- TL21 model does not support this method, TL51 supported.

Invoke example

LONG ret =0；

CString info;

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,100,225);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is under or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is above specified range，error code:%d",ret);

MessageBox(info);

break;

}

ret = Draw2DBarCodeTSPL(m_hPrinter,TSPL_BT_QRCODE,10,10,7,6,0,”www.test.com”);

switch(ret)

{

case TSPL_IDERROR:

info.Format("Draw2DBarCodeTSPL Parameter m_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

- info.Format("Draw2DBarCodeTSPL Parameter start_x < 0 or start_y < 0 or

- content==NULL，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

- info.Format("Draw2DBarCodeTSPL when max Parameter value is not among string x1 ~

- x6 range, then return this error，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_INVALID:

info.Format( "The value of the Draw2DBarCodeTSPL parameter is invalid. Error code:%d",

ret);

MessageBox(info);

return;

}

PrintTSPL(m_hPrinter,1,1);

### LONG  PutBitMapTSPL(LONG iPrinterID,INT start_x, INT start_y,CString fileName)

Function

Put downloaded bitmap to print buffer area

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- start_x: coordinate position in horizontal position, in dots(cannot exceed page width)

- start_y: coordinate position in vertical position, in dots(cannot exceed page height)

- fileName: Downloaded bitmap name in printer memory, must be same with bitmap downloaded in printer memory or flash, need to add suffix name and in capital format, otherwise probably cannot print out. DownLoadBitmap method downloaded bitmap in printer, then PutBitmap method put bitmap into printer buffer.

- Returned value

- True:TSPL_SUCCESS(0)

- Fault:TSPL_IDERROR(-1): Parameter iPrinterID  got error, return this error

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):  Parameter start_x < 0 or start_y < 0 or fileName== NULL

- Description

- TL21 model does not support this method for now, TL51 supported.

Invoke example

LONG ret =0；

CString info;

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

}

ret = PageSetupTSPL(m_hPrinter,100,225);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPLParameterPageWidth or PageHeight is less or equal to 0，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPL Parameter PageWidth or PageHeight is more than or equal to specified scope，error code:%d",ret);

MessageBox(info);

break;

}

ret = DownLoadBitMapTSPL(m_hPrinter,FALSE,”D://LOG.bmp”);

switch(ret)

{

case TSPL_IDERROR:

info.Format("DownLoadBitMapTSPLParameterm_hPrinter got error or failed to open file:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("DownLoadBitMapTSPLParameterPathName==NULL:%d",ret);

MessageBox(info);

break;

}

ret = PutBitMapTSPL(m_hPrinter,10,10,”D://LOG.BMP”);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PutBitMapTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PutBitMapTSPLParameterfileName==NULL:%d",ret);

MessageBox(info);

break;

}

PrintTSPL(m_hPrinter,1,1);

### LONG  DrawBitMapTSPL(LONG iPrinterID,INT start_x, INT start_y, INT mode, LPCSTR fileName)

Function

Draw bitmap

Parameter

- iPrinterID：Printer handle, is confirmed by returned value of opened port

- start_x: coordinate position in horizontal position, in dots(cannot exceed page width)

- start_y: coordinate position in vertical position, in dots(cannot exceed page height)

- mode: OVERWRITE、OR and XOR

- fileName：The bitmap name。

Returned value

- True：TSPL_SUCCESS(0)

- Fault：TSPL_IDERROR(-1): ParameteriPrinterID  got error, return this error

TSPL_PARAM_LESS_EQUAL_ZERO(-2): Parameterstart_x < 0 or start_y < 0 or fileName== NULL

Invoke example

LONG ret =0；

CString info;

ret = ClearBuffTSPL(m_hPrinter);

if(ret < 0)

{

info.Format("ClearBuffTSPLParameterm_hPrinter error:%d", ret);

MessageBox(info);

return;

}

ret = PageSetupTSPL(m_hPrinter,100,225);

switch(ret)

{

case TSPL_IDERROR:

info.Format("PageSetupTSPL Parameter m_hPrinter error:%d", ret);

MessageBox(info);

return;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format("PageSetupTSPL Parameter PageWidth or PageHeight is less or equal to 0，error

code:%d", ret);

MessageBox(info);

return;

case TSPL_PARAM_GREAT_RANGE:

info.Format("PageSetupTSPL Parameter PageWidth or PageHeight is more than or equal to

specified scope，error code:%d", ret);

MessageBox(info);

return;

}

ret = DrawBitMapTSPL(m_hPrinter,10,10,0,”D://LOG.bmp”);

switch(ret)

{

case TSPL_IDERROR:

info.Format( "DrawBitMapTSPL Parameter m_hPrinter error:%d", ret);

MessageBox(info);

return;

case TSPL_PARAM_LESS_EQUAL_ZERO:

info.Format( "DrawBitMapTSPL Parameter fileName==NULLor start_x<0 or start_y <

- error code:%d", ret);

MessageBox(info);

return;

}

PrintTSPL51(m_hPrinter,1,1);

### LONG  SetCharsetNameTSPL(LONG iPrinterID,CString CharsetName)

Function

Set international Characters Set

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- CharsetName: Optional character range is among below string: "U.S.A","France","Germany","U.K","Denmark","Sweden","Italy","Spain","Japan","Norway","Denmark","Spain","Latin","Korea","Slovenia","China"

- Returned value

- True: TSPL_SUCCESS(0)

- Fault: TSPL_IDERROR(-1): when ParameteriPrinterID  got error, return this error.

- TSPL_PARAM_LESS_EQUAL_ZERO(-2):  when CharsetNameParameter is NULL,  return this error

- TSPL_PARAM_GREAT_RANGE(-3): when CharsetNameParameter is not at above range, return this error

- Description

- TL21 model does not support this method, TL51 supported.

Invoke example

LONG ret =0；

CString info;

ret = SetCharsetNameTSPL(m_hPrinter,"U.S.A");

switch(ret)

{

case TSPL_IDERROR:

info.Format("SetCharsetNameTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_LESS_EQUAL_ZERO:

- info.Format("SetCharsetNameTSPL when CharsetNameParameter is NULL, return this error，error code:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

- info.Format("SetCharsetNameTSPL when CharsetNameParameter is not at above range, return this error，error code:%d",ret);

MessageBox(info);

break;

}

### LONG  SelectCodePageTSPL(LONG iPrinterID,INT value)

Function

Choose characters codepage

Parameter

- iPrinterID: Printer handle, is confirmed by returned value of opened port

- value: Choose the NO.n page from below character codes pages.

| N | 代码页 | Code Page |
| --- | --- | --- |
| 0 | CP437 [美国，欧洲标准] | CP437 [U.S.A., Standard Europe] |
| 1 | KataKana [片假名] | Katakana |
| 2 | PC850 [多语言] | PC850 [Multilingual] |
| 3 | PC860 [葡萄牙] | PC860 [Portuguese] |
| 4 | PC863 [加拿大-法语] | PC863 [Canadian-French] |
| 5 | PC865 [北欧] | PC865 [Nordic] |
| 6 | WCP1251 [斯拉夫语] | WCP1251 [Cyrillic] |
| 7 | CP866 斯拉夫2 | CP866 Cyrilliec #2 |
| 8 | MIK[斯拉夫/保加利亚] | MIK[Cyrillic /Bulgarian] |
| 9 | CP755 [东欧，拉脱维亚 2] | CP755 [East Europe，Latvian 2] |
| 10 | [伊朗，波斯] | Iran |
| 11 | 保留 | reserve |
| 12 | 保留 | reserve |
| 13 | 保留 | reserve |
| 14 | 保留 | reserve |
| 15 | CP862 [希伯来] | CP862 [Hebrew] |
| 16 | ＷCP1252 [拉丁语 1] | ＷCP1252 Latin I |
| 17 | WCP1253 [希腊] | WCP1253 [Greek] |
| 18 | CP852 [拉丁语 2] | CP852 [Latina 2] |
| 19 | CP858 [多种语言拉丁语 1+欧元符] | CP858 Multilingual Latin Ⅰ+Euro) |
| 20 | 伊朗Ⅱ[波斯语] | Iran II |
| 21 | 拉脱维亚 | Latvian |
| 22 | CP864 [阿拉伯语] | CP864 [Arabic] |
| 23 | ISO-8859-1 [西欧] | ISO-8859-1 [West Europe] |
| 24 | CP737 [希腊] | CP737 [Greek] |
| 25 | WCP1257 [波罗的海] | WCP1257 [Baltic] |
| 26 | [泰文1] | Thai 1 |
| 27 | CP720[阿拉伯语] | CP720[Arabic] |
| 28 | CP855 | CP855 |
| 29 | CP857[土耳其语] | CP857[Turkish] |
| 30 | WCP1250[中欧] | WCP1250[Central Eurpoe] |
| 31 | CP775 | CP775 |
| 32 | WCP1254[土耳其语] | WCP1254[Turkish] |
| 33 | WCP1255[希伯来语] | WCP1255[Hebrew] |
| 34 | WCP1256[阿拉伯语] | WCP1256[Arabic] |
| 35 | WCP1258[越南语] | WCP1258[Vietnam] |
| 36 | ISO-8859-2[拉丁语2] | ISO-8859-2[Latin 2] |
| 37 | ISO-8859-3[拉丁语3] | ISO-8859-3[Latin 3] |
| 38 | ISO-8859-4[波罗的语] | ISO-8859-4[Baltic] |
| 39 | ISO-8859-5[斯拉夫语] | ISO-8859-5[Cyrillic] |
| 40 | ISO-8859-6[阿拉伯语] | ISO-8859-6[Arabic] |
| 41 | ISO-8859-7[希腊语] | ISO-8859-7[Greek] |
| 42 | ISO-8859-8[希伯来语] | ISO-8859-8[Hebrew] |
| 43 | ISO-8859-9[土耳其语] | ISO-8859-9[Turkish] |
| 44 | ISO-8859-15[拉丁语9] | ISO-8859-15 [Latin 3] |
| 45 | [泰文2] | Thai2 |
| 46 | CP856 | CP856 |

- Returned value

- True: TSPL_SUCCESS(0)

- Fault: TSPL_IDERROR(-1):  when ParameteriPrinterID got error, return this error.

- TSPL_PARAM_GREAT_RANGE(-3): when valueParameter is not at above range, return this error

- Description

- TL51 model does not support this method now,TL21 supported

Invoke example

LONG ret =0；

CString info;

SetIs21();//TL21 need to use this method

ret = SelectCodePageTSPL(m_hPrinter,21);

switch(ret)

{

case TSPL_IDERROR:

info.Format("SelectCodePageTSPLParameterm_hPrinter error:%d",ret);

MessageBox(info);

break;

case TSPL_PARAM_GREAT_RANGE:

- info.Format("SelectCodePageTSPL when CharsetNameParameter is not at above range, return this error，error code:%d",ret);

MessageBox(info);

break;

}

## 2.10  CPCL Function illustration

### LONG CPCL_Print(LONG iPrinterID)

Function

CPCL print command

Parameter

- iPrinterID: printer handle is decided by returned value which open port

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Illustration

- In the CPCL command mode, the print command terminates and prints the file, which is the last command; after the print command is executed, the printer exits the control section, and a carriage return and line feed must be followed after the print command. If an error occurs during printing, wait for the error to recover and then print again.

### LONG CPCL_Form(LONG iPrinterID)

Function

CPCL Find the gap command

Parameter

- iPrinterID: printer handle is decided by returned value which open port

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Illustration

- After using the FORM command, the printer will feed the paper to the starting position of the next label. The starting position of the printer to find the next label is judged according to the gap between the two labels.

### LONG CPCL_PageSetup(LONG iPrinterID,INT offset, INT height, INT qty, INT width)

Function

Set parameters such as label size

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- offset: offset relative to the zero point of the nose

- height: the maximum height of the label, in dots

- qty: the number of printed labels

- width: The maximum width of the label, in dots.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Illustration

- The width parameter is usually not set, and width=0 means not set. But if you need to rotate, that is, call CPCL_SetPageRotate, you must set this parameter, otherwise the rotation will not work.

Invoke example

ret = CPCL_PageSetup(m_hPrinter,0,800,1,0);

switch(ret)

{

case CPCL_IDERROR:

info.Format(“%s function parameter m_hPrinter error:%d”, "PageSetup",ret);

MessageBox(info);

return;

case CPCL_PARAM_INVALID:

info.Format(“%sfunction parameter is invalid,error code:%d”,"PageSetup",ret);

MessageBox(info);

return;

case CPCL_OVERTIME:

info.Format(“%s  function read and write timeout, error code:%d”, "PageSetup",ret);

MessageBox(info);

return;

case CPCL_OTHERERRORS:

info.Format(“%s function returned other errors:%d”, "PageSetup",ret);

MessageBox(info);

return;

case CPCL_WRITEFAIL:

info.Format(“%s Failed to write the %s function. Error code:%d”, "PageSetup",ret);

MessageBox(info);

return;

}

### LONG CPCL_DrawLine(LONG iPrinterID, INT x0, INT y0, INT x1, INT y1, INT width)

Function

Line drawing commands.

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- x0: The horizontal (x) coordinate of the upper left corner

- y0: The vertical (y) coordinate of the upper left corner

- x1: The horizontal (x) coordinate of the upper right corner

- The vertical horizontal (x) coordinate of the lower left corner

- y1: The vertical (y) coordinate of the upper right corner

- The vertical (y) coordinate of the lower left corner

- width: line width

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

//line drawing

CPCL_DrawLine(m_hPrinter,10,10,100,100,2);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_DrawBox(LONG iPrinterID, INT x0, INT y0, INT x1, INT y1, INT width)

Function

Frame commands

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- x0: Top left corner x coordinate

- y0: Top left corner y coordinate

- x1: Bottom right corner x coordinate

- y1: Bottom right corner y coordinate

- width: Frame width

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

//draw rectangle

CPCL_DrawBox(m_hPrinter,10,10,100,100,2);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG  CPCL_DrawText(LONG iPrinterID, INT value, INT font, INT vsize, INT hsize, INT x, INT y, LPCSTR data)

Function

Text commands

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- Value: print text direction.

- 0: level;

- 1: Rotate 90 degrees counterclockwise;

- 2: Rotate 180 degrees counterclockwise;

- 4: Rotate 270 degrees counterclockwise.

- Font: Select the font size. Value is 0 or 1.

- 0 corresponds to font 24, indicating characters (12*24), Chinese characters (24*24)

- 1 corresponds to font 55, representing characters (8*16), Chinese characters (16*16)

- Vsize: Vertical magnification. The range of values is 0~8

- Hsize: Horizontal magnification. The range of values is 0~8

- x: The starting position of horizontal printing.

- y: vertical print start position

- data: The printed text content.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

//draw text print horizontally

CPCL_DrawText(m_hPrinter,0,0,2,2,10,110,”Printer test”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

CPCL_DrawText(m_hPrinter,1,0,3,3,10,600,”Printer test”); //90 degree rotation printing

CPCL_DrawText(m_hPrinter,2,0,1,1,1000,600,”Printer test”); //180 degree rotation printing

CPCL_DrawText(m_hPrinter,3,0,1,1,100,200,”Printer test”); //270 degree rotation printing

### LONG CPCL_InverseLine(LONG iPrinterID, INT x0, INT y0, INT x1, INT y1, INT width)

Function

Invert command

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- x0: The X coordinate of the upper left corner.

- y0: Y coordinate of the upper left corner

- x1: The horizontal X coordinate of the upper right corner.

- The vertical X coordinate of the lower left corner

- y1: The horizontal Y coordinate of the upper right corner.

- The vertical Y coordinate of the lower left corner.

- Width: Invert content width

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Illustration

- In the area selected by the inverse display, the content generated in the area is drawn in white, and the white area is drawn in black.

Print Effect

- CPCL_PageSetup(m_hPrinter,0,800,1,0);

- // draw text

- CPCL_DrawText(m_hPrinter,0,0,2,2,10,300,”Printer test”);

- //Invert

- CPCL_InverseLine(m_hPrinter,10,300,800,300,90);

- CPCL_Form(m_hPrinter);

- CPCL_Print(m_hPrinter);

### LONG CPCL_SetAlign(LONG iPrinterID, INT align)

Function

Set the alignment of the current line content

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- align: 0: left; 1: center; 2: right.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Invoke example

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_SetAlign(m_hPrinter,0);  //left

//Draw text

CPCL_DrawText(m_hPrinter,0,0,1,1,10,10,”printer test”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

CPCL_SetAlign(m_hPrinter,1);  //center

CPCL_SetAlign(m_hPrinter,2);  //right

### LONG CPCL_SetBold(LONG iPrinterID, INT value)

Function

Set font bold

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- Value: 1: bold; 0: cancel bold.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_SetBold(m_hPrinter,1);  //bold

//draw text

CPCL_DrawText(m_hPrinter,0,0,1,1,10,10,”printer test”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_SetInverseText(LONG iPrinterID, INT value)

Function

Set the characters to be highlighted

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- Value: Indicates whether to highlight characters

- 1: Inverted display;

- 0: Cancel the reverse display.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_SetInverseText(m_hPrinter,1);

//draw text

CPCL_DrawText(m_hPrinter,0,0,1,1,10,10,”printer test”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_SetSpacing(LONG iPrinterID, INT spacing)

Function

Set character spacing

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- spacing: The distance between characters, the default character spacing is 0.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_SetSpacing(m_hPrinter,2);

//draw text

CPCL_DrawText(m_hPrinter,0,0,1,1,10,10,”printer test”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_SetUnderLineText(LONG iPrinterID, INT value)

Function

Set whether to print underline

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- Value: The value is 0, 1, 2.

- 0: Cancel printing underline;

- 1: print 1 dot underline;

- 2: print 2 dots underline;

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_SetUnderLineText(m_hPrinter,1); //1 dot underline

//draw text

CPCL_DrawText(m_hPrinter,0,0,1,1,10,10,”printer test”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

CPCL_SetUnderLineText(m_hPrinter,2); //2 dots underline

CPCL_SetUnderLineText(m_hPrinter,0); //no underline

### LONG CPCL_DrawBarcode(LONG iPrinterID, INT value, LPCSTR type, INT width, INT ratio, INT height, INT x, INT y, LPCSTR data)

Function

Printing 1D barcodes

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- Value: print barcode orientation.

- 0: Print horizontal barcode

- 1: Print vertical barcodes

- type: barcode type.

- type value   barcode type

- UPCA       UPC-A

- UPCE       UPC-E

- EAN13      JAN13 (EAN13)

- EAN8       JAN 8 (EAN8)

- 39          CODE39

- CODABAR   CODABAR

- 93          CODE93

- 128         CODE128(Auto)

- width: the width of the narrow bar of the barcode

- ratio: the ratio of the width of the wide bar and the width of the narrow bar, the range is 0~30

- height: barcode height

- x: the starting position of the barcode in the horizontal direction

- y: the starting position of the barcode in the vertical direction

- data: barcode content

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_DrawBarCode(m_hPrinter,0,”128”,2,1,30,10,10,”NO.123456”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_SetHRI(LONG iPrinterID, LPCSTR offset)

Function

HRI character command (set 1D barcode text to print automatically)

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- offset: The upper and lower relative displacement of the HRI character and the barcode. Value is a numeric string or "OFF".

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_SetHRI(m_hPrinter,”50”);

CPCL_DrawBarCode(m_hPrinter,0,”128”,2,1,30,10,10,”NO.123456”);

CPCL_SetHRI(m_hPrinter,”OFF”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG  CPCL_Draw2Barcode_QR(LONG iPrinterID, INT x, INT y, INT Mn, INT Un,  LPCSTR Sn,  LPCSTR data)

Function

Print QR code

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- x: The starting position of the barcode in the horizontal direction.

- y: The starting position in the vertical direction of the barcode.

- Mn: QR barcode mode. Value is 1 or 2, default is 2.

- Un: magnification. The range of values is 1~6, the default is 6.

- Sn: Error correction level.

- Sn value Corresponding grade

- H Ultra-high reliability level

- Q high reliability level

- M standard grade

- L high density grade

- data: barcode data

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_Draw2Barcode_QR(m_hPrinter,10,100,2,6,”M”,”www.test.com”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_Draw2Barcode_PDF417(LONG iPrinterID, INT x, INT y, INT XDn, INT YDn,  INT Cn, INT Sn, LPCSTR content)

Function

Print PDF417 2D barcode

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- x: The starting position of the barcode in the horizontal direction.

- y: The starting position in the vertical direction of the barcode.

- XDn: Minimum unit horizontal width. The range of values is 1~32, the default is 2.

- YDn: Minimum unit vertical height. The range of values is 1~32, the default is 6.

- Cn: Number of characters per line. The range of values is 1~30, the default is 3.

- Sn: Error correction level. The range of values is 1~8, the default is 1.

- data: barcode data

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_Draw2Barcode_PDF417(m_hPrinter,10,20,3,12,3,2,”www.test.com”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_PrintBMP(LONG iPrinterID, INT type,INT x, INT y, LPCSTR lpFilePath)

Function

Printing Monochrome Bitmaps

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- type: A value of 0 or 1.0 means printing extended graphics; 1 means printing compressed graphics

- x: The starting position in the horizontal direction.

- y: The starting position in the vertical direction.

- lpFilePath: Image path and file name.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,0);

CPCL_PrintBMP(m_hPrinter,1,10,20,”E://test.bmp”);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

### LONG CPCL_SetPageRotate(LONG iPrinterID, INT value)

Function

Rotate the entire page 90 degrees clockwise

Parameter

- iPrinterID: printer handle is decided by returned value which open port

- value: Indicates whether the entire page is rotated 90 degrees clockwise

- 1: rotate;

- 0: Do not rotate.

Returned value

- True: CPCL_SUCCESS(0)

- False:

- CPCL_IDERROR(-1): This error is returned when the parameter iPrinterID is wrong.

- CPCL_WRITEFAIL(-2): Printer write failed

- CPCL_PARAM_INVALID(-4): Invalid parameter

- CPCL_OVERTIME(-5): timeout

- CPCL_OTHERERRORS(-100): other errors

Illustration

- When calling the CPCL_SetPageRotate function, you need to set the width parameter in the CPCL_PageSetup function, otherwise it will not work

Print Effect

CPCL_PageSetup(m_hPrinter,0,800,1,100);

CPCL_PrintBMP(m_hPrinter,1,10,400,”E://test.bmp”);

CPCL_SetPageRotate(m_hPrinter,1);

CPCL_Form(m_hPrinter);

CPCL_Print(m_hPrinter);

# Appendix

- CODE128	summarize

CODE128 can use character set A、character set B and character set C interchangeably, and can encode 128 characters of ASCII, 100 numbers 00~99 and some special characters. The code character of each character set are below:

· Character set A: ASCII characters 00H~5FH

· Character set B: ASCII characters 20H~7FH

· Character set C: 100 numbers 00~99

CODE128 can encode below special character

· SHIFT Character

“SHIFT” can convert the first character after SHIFT character from character set A to character set B or from character set B to character set A. From the second character, it will return to the character set which is used by SHIFT previously. “SHIFT”characters can only be used alternatively between character set A and character set B. It can’t make the encoding character enter or exit the status of character set C.

·Character set chosen character（CODE A、CODE B、CODE C）

These characters can convert the rear code characters to character A、B or C

· Function character（FNC1、FNC2、FNC3、FNC4）

The use of functional characters depends on the application software. In character set C, only FNC1 is available.

- 2. Character set

Character set A

| character | send data |  | character | send data |  | character | send data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hex | Decimal |  | Hex | Decimal |  | Hex | Decimal |
| NULL SOH STX ETX EOT ENQ ACK BEL BS HT LF VT FF CR SO SI DLE DC1 DC2 DC3 DC4 NAK SYN ETB CAN EM SUB ESC FS GS RS US SP ! " # $ % | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F 20 21 22 23 24 25 | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 | & ' ( ) * + , - . / 0 1 2 3 4 5 6 7 8 9 : ; < = > ? @ A B C D E F G H I J K | 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 46 47 48 49 4A 4B | 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 | L M N O P Q R S T U V W X Y Z [ \ ] ^ _ FNC1 FNC2 FNC3 FNC4 SHIFT CODEB CODEC | 4C 4D 4E 4F 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 7B,31 7B,32 7B,33 7B,34 7B,53 7B,42 7B,43 | 76 77 78 49 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 123,49 123,50 123,51 123,52 123,83 123,66 123,67 |

Character set B

| character | send data |  | character | send data |  |  | character | send data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hex | Decimal |  | Hex | Decimal |  |  | Hex | Decimal |
| SP ! " # $ % & ' ( ) * + , - . / 0 1 2 3 4 5 6 7 8 9 : ; < = > ? @ A B C D E | 20 21 22 23 24 25 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 | 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 | F G H I J K L M N O P Q R S T U V W X Y Z [ \ ] ^ _ ` a b c d e f g h i j k | 46 47 48 49 4A 4B 4C 4D 4E 4F 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 60 61 62 63 64 65 66 67 68 69 6A 6B |  | 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 | l m n o p q r s t u v w x y z { ¦ } — DEL FNC1 FNC2 FNC3 FNC4 SHIFT CODEA CODEC | 6C 6D 6E 6F 70 71 72 73 74 75 76 77 78 79 7A 7B,7B 7C 7D 7E 7F 7B,31 7B,32 7B,33 7B,34 7B,53 7B,41 7B,43 | 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123,123 124 125 126 127 123,49 123,50 123,51 123,52 123,83 123,65 123,67 |

Character set C

| character | send data |  | Character | send data |  |  | character | send data |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hex | Decimal |  | Hex | Decimal |  |  | Hex | Decimal |
| 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 | 00 01 02 03 04 05 06 07 08 09 0A 0B 0C 0D 0E 0F 10 11 12 13 14 15 16 17 18 19 1A 1B 1C 1D 1E 1F 20 21 22 23 24 25 | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 | 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 | 26 27 28 29 2A 2B 2C 2D 2E 2F 30 31 32 33 34 35 36 37 38 39 3A 3B 3C 3D 3E 3F 40 41 42 43 44 45 46 47 48 49 4A 4B |  | 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 | 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 FNC1 CODEA CODEB | 4C 4D 4E 4F 50 51 52 53 54 55 56 57 58 59 5A 5B 5C 5D 5E 5F 60 61 62 63 7B,31 7B,41 7B,42 | 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 123,49 123,65 123,66 |